/*
Automated fluorescence overlap analysis for macrophage phagocytosis assay

Software:
QuPath (version ___)

Input:
RGB fluorescence PNG images containing:
- Red fluorescent macrophages
- Green fluorescent tumor cells

Output:
CSV file containing:
- Red macrophage pixel area
- Green tumor pixel area
- Green signal within macrophage regions
- Phagocytosis index (%)

Calculation:
Phagocytosis index (%) =
(Green-positive pixels within red macrophage regions /
 Total red macrophage pixels) × 100
*/


import qupath.lib.gui.scripting.QPEx
import qupath.lib.regions.RegionRequest
import java.awt.image.BufferedImage


def project = QPEx.getProject()

if (project == null) {
    println "No QuPath project detected"
    return
}


def results = []


// Analyze only phagocytosis assay images

for (entry in project.getImageList()) {

    def imageName = entry.getImageName()

    if (!imageName.toLowerCase().contains("phag"))
        continue


    println "Analyzing: " + imageName


    def imageData = entry.readImageData()
    def server = imageData.getServer()


    def request = RegionRequest.createInstance(
        server.getPath(),
        1,
        0,
        0,
        server.getWidth(),
        server.getHeight()
    )


    BufferedImage img = server.readBufferedImage(request)


    long macrophagePixels = 0
    long tumorPixels = 0
    long tumorSignalInsideMacrophages = 0



    // Pixel-based fluorescence classification

    for (int y = 0; y < img.getHeight(); y++) {

        for (int x = 0; x < img.getWidth(); x++) {


            int rgb = img.getRGB(x,y)

            int red   = (rgb >> 16) & 0xff
            int green = (rgb >> 8) & 0xff
            int blue  = rgb & 0xff



            // Red-positive macrophage signal

            boolean macrophage =
                    red > 50 &&
                    red > green*1.4 &&
                    red > blue*1.4



            // Green-positive tumor signal

            boolean tumor =
                    green > 35 &&
                    green > red*0.35 &&
                    green > blue*1.1



            if (macrophage)
                macrophagePixels++


            if (tumor)
                tumorPixels++


            // Tumor fluorescence localized within macrophage area

            if (macrophage && tumor)
                tumorSignalInsideMacrophages++

        }
    }



    double phagocytosisIndex = 0


    if (macrophagePixels > 0) {

        phagocytosisIndex =
        (tumorSignalInsideMacrophages /
        (double) macrophagePixels) * 100

    }



    results << [
        imageName,
        macrophagePixels,
        tumorPixels,
        tumorSignalInsideMacrophages,
        phagocytosisIndex
    ]

}



// Export results

def output = buildFilePath(
    PROJECT_BASE_DIR,
    "phagocytosis_analysis_results.csv"
)



new File(output).withWriter { writer ->


    writer.writeLine(
    "Image,Macrophage_pixels,Tumor_pixels,Tumor_signal_inside_macrophages,Phagocytosis_index_percent"
    )


    results.each { row ->

        writer.writeLine(
        "${row[0]},${row[1]},${row[2]},${row[3]},${row[4]}"
        )

    }

}


println "Analysis complete"
println "Results saved to:"
println output